Network Wizards Cellular Telephone Library Routines

Copyright (C) 1994 by Network Wizards

USING THE PROGRAMMING LIBRARIES

The programming library file is CTLIB.OBJ.  The module and all other code

were written in Borland Turbo C 2.0 using the small memory model, and word

alignment (switches "-a -ms").  See the enclosed Makefile for compiling

examples.  Any programs you write need to include CTLIB.H, and be linked

with CTLIB.OBJ, TTY.OBJ, and TTYA.OBJ.  If you have trouble linking these

with other compilers, please contact us for help.

[Note: There is a bug in the delay() function of Turbo C which may cause

programs to not work on 486 machines.  If you have this problem you can get

a patch file from the Borland BBS at (408) 439-9096]

CTLIB.H contains function header prototypes, structure element descriptions,

and predefined constants.

TTY.OBJ and TTYA.OBJ contain serial I/O code used by CTLIB.OBJ to talk

to the COM port the interface adapter is attached to.

The function descriptions use the following data types:

typedef unsigned char bool;

typedef unsigned char byte;

typedef unsigned int  word;  /* 16 bits */

======================================================================

FUNCTION SUMMARY

INITIALIZATION

ct_lib_init			initialize library routines

ct_lib_done			cleanup after library routines

ct_on				turn phone on

ct_off				turn phone off

OPERATION

ct_keypress			"press" a key on phone

TRANSMISSION

ct_set_channel			select channel number

ct_set_carrier			turn transmitter on or off

ct_set_tx_power			set transmit power level

ct_set_sat			select sat tones

AUDIO

ct_set_audio_path		set source/destination of audio signals

ct_set_rx_audio			mute or unmute received audio

ct_set_tx_audio			mute or unmute transmitted audio

ct_set_volume			set audio volume level

ct_set_audio_tone		select audio alerting tones

ct_set_dtmf			transmit DTMF tones

ct_dtmf_decode			receive DTMF tones

ct_set_comp_exp			turn compressor/expandor circuit on or off

DATA AND SIGNALLING

ct_fcc_msg			receive a Forward Control Channel message

ct_fvc_msg			receive a Forward Voice Channel message

ct_rcc_msg			send a Reverse Control Channel message

ct_rvc_msg			send a Reverse Voice Channel message

ct_set_signalling_tone		turn signalling tone on or off

ct_stream			set reception on even or odd data stream

ct_get_bi			get busy/idle status

STATUS

ct_get_rss			read received signal strength

ct_get_battery_level		read battery level

MEMORIES

ct_read_mem			read telephones internal memory

ct_read_block			read memory block from phone

ct_set_block			write memory block to phone

ct_get_esn			read ESN from phone

ct_get_romver			read ROM version number from phone

ct_get_prog_pwd			read programming password from phone

ct_get_cust_msg			read custom power on message from phone

ct_set_cust_msg			write custom power on message to phone

ct_get_nams			read NAMs out of phone

ct_set_nams			write NAMs into phone

UTILITIES

ct_inc_channel			increment to next channel

ct_dec_channel			decrement to next channel

ct_channel_type			return type of channel

ct_decode_fcc_msg		decode a forward control channel message

ct_decode_fvc_msg		decode a forward voice channel message

ct_decode_order			decode a received order

ct_tel2min			convert a telephone number to a MIN

ct_gen_bch			generate a BCH error correction code

======================================================================

PHONE CONTROL FUNCTIONS

int ct_lib_init(model,addr,irq)

word model;

word addr;

byte irq;

Initializes the cellular library routines and serial I/O routines.

This routine must be called only once before any other routine

may be used.

model is the cellular telephone model number to be accessed, one of:

model	Description

900	OKI-900 or AT&T-3730

1150	OKI-1150 or AT&T-3760

[either model number can be used and ctlib will figure out what

type of phone it is really talking to]

addr is the I/O address of the serial port UART.

irq is the interrupt request line of the UART.

addr and irq are typically set as follows:

port	(addr,irq)

COM1	(0x3f8,4)

COM2	(0x2f8,3)

Returns TRUE on success.

void ct_lib_done()

Cleans up the cellular and serial I/O routines.

This routine must be called before exiting your program.

int ct_on(mode)

word mode;

Power up the phone in normal or test/debug mode.

mode is one of MODE_NORMAL or MODE_TEST.

All routines in this library function only in MODE_TEST,

except for ct_keypress() which only works in MODE_NORMAL.

Returns TRUE on success.

void ct_off()

Power down the phone.

void ct_keypress(key)

int key;

This function may only be used if the phone is powered up in

MODE_NORMAL.  A key is "pressed" by sending its keypress code

defined in ctlib.h.  A key is "released" by sending the release code.

void ct_set_carrier(state)

bool state;

Turns the transmitter on (TRUE) or off (FALSE).

void ct_set_tx_power(level)

int level;

Sets the transmitter power attenuation level, zero being the

highest power level (from 0-7).   See cellular specs for definition

of the attenuation levels.

void ct_set_sat(sat)

int sat;

Controls the transmission of SAT tones.  sat is 0 thru 3 as follows:

SAT		Operation

0		transmit 5970hz SAT

1		transmit 6000hz SAT

2		transmit 6030hz SAT

3		disable transmission of SAT tones

void ct_set_dtmf(tone)

byte tone;

Begins transmission of a DTMF tone.  Tone is one of the standard values

represented by "1234567890*#ABCD".  If tone is an ASCII space, DTMF

transmission is stopped.  The following special tones "efghijkl" are

defined as follows:

e	697hz (row1)		i	1209hz (col1)

f	770hz (row2)		j	1336hz (col2)

g	852hz (row3)		k	1477hz (col3)

h	941hz (row4)		l	1633hz (col4)

void ct_set_signalling_tone(state)

bool state;

Begins transmitting a 10khz signalling tone (TRUE) or stops it (FALSE).

void ct_set_audio_tone(tone,state)

int tone;

bool state;

Turns a tone on (TRUE) or off (FALSE).  The tone is sent to the

current received audio path.  The tone is one of:

Tone Value	Frequency

TONE_LOW	770hz

TONE_HIGH	1150hz

void ct_set_rx_audio(state)

bool state;

Mutes (FALSE) or Unmutes (TRUE) the received audio signal.

void ct_set_tx_audio(state)

bool state;

Mutes (FALSE) or Unmutes (TRUE) the transmitted audio signal.

void ct_set_comp_exp(state)

bool state;

Enables (TRUE) or disables (FALSE) the audio compressor and expandor

circuits.

void ct_set_volume(vol)

int vol;

Sets the audio output volume (0-7).  0 is the highest level.

void ct_set_audio_path(path)

int path;

Sets the audio path to and from the radio.  One of:

Path			Description

AUDIO_EXTERNAL	audio routed to external jack

AUDIO_EARPIECE	input from mic, output to earpiece

AUDIO_SOUNDER		input from mic, output to earpiece and sounder

void ct_set_channel(channel)

word channel;

Tunes the cellular phone to the specified channel (1-1023).

int ct_dtmf_decode(command)

int command;

Decodes DTMF tones received from the radio.  No other commands may

be sent to the phone between START and END of decoding period.

command is one of:

DTMF_DECODE_START	Start listening for DTMF tones (always returns TRUE).

DTMF_DECODE_GET	Returns FALSE if no valid tones received so far,

or the ASCII value of one of "1234567890*#ABCD".

DTMF_DECODE_END	End listening for DTMF tones (always returns TRUE).

int ct_fvc_msg(command,data)

int command;

byte *data;

Receives a message sent on the Forward Voice Channel, from the land

station to mobile station.  MSG_SETUP must be called to get the phone

ready to receive the next message.  MSG_GET can then be called to find

out when it arrives.  If a message is not received, MSG_ABORT can be

used to end the MSG_SETUP waiting mode.  No other commands can be issued

to the phone if it is waiting for a message.

command is one of:

MSG_SETUP		Start listening for the next FVC message on the

current	voice channel.

MSG_GET		Returns FALSE if no messages have been received

so far, or TRUE and the message in the location

pointed to by data.

MSG_ABORT		Abort listening for FVC messages.

data is NULL for MSG_SETUP and MSG_ABORT.  For MSG_GET, data points to

a 5 byte array to put the received message into.

int ct_fcc_msg(command,data)

int command;

byte *data;

Receives a message sent on the Forward Control Channel, from the land

station to mobile station.  MSG_SETUP must be called to get the phone

ready to receive the next message.  MSG_GET can then be called to find

out when it arrives.  If a message is not received, MSG_ABORT can be

used to end the MSG_SETUP waiting mode.  No other commands can be issued

to the phone if it is waiting for a message.

command is one of:

MSG_SETUP		Start listening for the next FCC message on the

current control channel.

MSG_GET		Returns FALSE if no messages have been received

so far, or TRUE and the message in the location

pointed to by data.

MSG_ABORT		Abort listening for FCC messages.

data is NULL for MSG_SETUP and MSG_ABORT.  For MSG_GET, data points to

a 10 byte array to put the received message into.

int ct_rcc_msg(dcc,data)

int dcc;

byte *data;

Sends a Reverse Control Channel message, from the mobile to the land

station.  dcc is the Digital Color Code received from the land

station (0-3).  data is a 30 byte array, consisting of five 6-byte words.

Returns TRUE if dcc and ESN (in data) are valid.

int ct_rvc_msg(data)

byte *data;

Sends a Reverse Voice Channel message, from the mobile to the land

station.  data is a 12-byte array, consisting of two 6-byte words.

Always returns TRUE.

int ct_get_rss()

Returns signal strength of currently tuned channel.

Value is adjusted for approximate range of 0-99 (99 is strongest).

-1 is returned on error.

int ct_get_battery_level(volts)

float *volts;

Returns battery level (0-8).  If volts is not NULL, it is set to

the estimated battery voltage.

int ct_read_mem(dest,addr,n)

byte *dest;

word addr;

int n;

Reads a contiguous block of memory from the cellular phone.  dest points

to a destination area where n bytes will be placed.  The block is read from

the cellular phones internal memory space starting at addr.  Returns TRUE

on success, FALSE otherwise.

int ct_read_block(dest,addr,n)

byte *dest;

word addr;

int n;

Reads a block of memory from the cellular phone.  dest points to a

destination area where n bytes will be placed.  The block is read from

the cellular phones internal memory space starting at addr, but not

necessarily incrementing addr by one for the next byte, because some

phones skip over certain protected addresses in the memory space.

Returns TRUE on success, FALSE otherwise.

int ct_get_esn(esn)

unsigned long *esn;

Places the cellular phones Electronic Serial Number (32-bits) into

the unsigned long pointed to by esn.  Returns TRUE on success,

FALSE otherwise.

char *ct_get_romver()

Returns a pointer to an internal string identifying the cellular

telephone ROM version.

char *ct_get_prog_pwd()

Returns a pointer to an internal string identifying the cellular

telephone programming password.

char *ct_get_cust_msg()

Returns a pointer to an internal string which is displayed by the

cellular telephone as a custom power on message.

int ct_set_cust_msg(msg)

byte *msg;

Given a string, sets the custom power on message displayed by the

cellular phone.  Returns TRUE if ok, FALSE otherwise.

int ct_get_nams()

Reads NAM information from the cellular telephone.  Returns TRUE if ok.

The NAM information is placed in the following data structure defined

in ctlib.h:

struct nam_data {

char number[15]			;Tel Num: "(nnn) nnn-nnnn"

char sid[6]				;System Id: nnnnn

char sys				;System: 'A' or 'B'

char ipch[5]			;Initial Paging Channel: nnnn

char olc[3]				;Overload Class: nn

char gim[3]				;Group ID Mark: nn

} nam_info[5]				;  for each of 5 NAMs

int ct_set_nams()

Set NAM information into the cellular telephone.  Returns TRUE if ok.

The NAM information from the data structure defined above is

written into the cellular phone.  The data must be formatted the same

way as ct_get_nams() returns it.  The 'sys' char is not used when

writing.

This routine must not be interrupted or an invalid NAM block will

be sent to the phone causing it to display an error when the phone is

returned to normal operating mode.  It may also be wise to use

ct_get_battery_level() before calling this function and make sure

it is at a reasonable level and not about to die, as this function

will take a little while to complete.  The ct_get_nams() function

should be called at least once before changing any information and

using ct_set_nams().  If ct_get_nams() has not been called, then

ct_set_nams() will return FALSE.

int ct_stream(stream)

int stream;

Sets or returns the status of whether the transceiver is receiving

on the even or odd data stream of the control channel.  If stream

is -1, then the status is returned (0 for even stream, 1 for odd).

If stream is 0 or 1, then the stream is changed to correspond to

the setting, and its new status is returned (or old status if it

couldn't switch).

NOTE:  Older versions of this function required at least one NAM

be programmed as even and one as odd.  This is no longer required.

int ct_get_bi()

Returns the status of the busy/idle bit when tuned to a valid

control channel.  Returns FALSE if busy and TRUE if idle.  This

bit indicates the status of the reverse control channel at the

instant it was polled, and is already invalid by the time this

function reports it.

struct ct_state_struct {

bool carrier;

bool sigtone;

bool rxaudio;

bool txaudio;

bool compexp;

int  powerlevel;

int  sat;

int  volume;

int  audiopath;

int  channel;

int  stream;

int  namindex;

int  oldnam;

} ct_state;

This structure contains internal state kept by the library routines.

It may be read but should not be changed.

======================================================================

UTILITY ROUTINES

word ct_inc_channel(channel)

word channel;

Given a channel number, returns next voice channel number in the

current system (A/B), then the next system, then wraps around.

word ct_dec_channel(channel)

word channel;

Given a channel number, returns previous voice channel number in the

current system (A/B), then the next system, then wraps around.

int ct_channel_type(channel,system,type)

word channel;

byte *system;

byte *type;

Given a channel number, returns two bytes indicating:

system

'A'	system A

'B'	system B

type

'A'	audio channel

'C'	control channel

Returns TRUE if valid channel number was given, FALSE otherwise.

int ct_decode_fvc_msg(msg,scc,pscc,order,ordq,local,chan,vmac)

byte *msg;

int *scc,*pscc,*order,*ordq,*local,*chan,*vmac;

Given a pointer to a 5-byte Forward Voice Channel message in msg,

it decodes the message into the listed fields.  Returns TRUE if OK,

or FALSE if invalid message.  If scc is 0, 1, or 2, then pscc,

chan, and vmac are returned.  Otherwise scc is 3 (invalid scc),

and pscc, order, ordq, and local are returned.

scc		SAT color code for new channel (if 0, 1, or 2)

pscc		present SAT color code of current channel

order		order type

ordq		order qualifier

local		local control field (when order is local control type)

chan		new voice channel number

vmac		new voice channel mobile attenuation code

int ct_decode_fcc_msg(msg,scc,dcc,min,order,ordq,local,chan,vmac)

byte *msg, **min;

int *scc,*dcc,*order,*ordq,*local,*chan,*vmac;

Given a pointer to a 10-byte Forward Control Channel message in msg,

it decodes the message into the listed fields.  Returns TRUE if OK,

or FALSE if invalid message.  If TRUE, always returns scc, dcc and min.

If scc is 0, 1, or 2, then chan, and vmac are returned.  Otherwise scc

is 3 (invalid scc), and order, ordq, and local are returned.

scc		SAT color code for new channel (if 0, 1, or 2)

dcc		digital color coded field

min		returns a pointer to a string with the Mobile Id

Number formatted as "(nnn) nnn-nnnn".  The string

is overwritten on each call.

order		order type

ordq		order qualifier

local		local control field (when order is local control type)

chan		voice channel number

char *ct_decode_order(order,ordq)

int order, ordq;

Given an order and order qualification code (such as those returned

from ct_decode_fvc_msg() and ct_decode_fcc_msg(), returns a pointer

to a string describing the order.  The string pointed is overwritten

on each call.

void ct_tel2min(telnum,min1,min2)

char *telnum;

unsigned long *min1, *min2;

Given a phone number in "(nnn) nnn-nnnn" format, it is returned

encoded as min1 and min2 as defined by the cellular specifications.

word ct_gen_bch(bits,buf)

int bits;

unsigned char *buf;

Generates a BCH error correcting code.  Takes number of bits to

generate a bch code for, and pointer to first byte of data buffer

containing the data bits.  Deposits 12-bit BCH code into buffer

right after the given number of data bits.