The following paper was originally published in the

Proceedings of the Sixth USENIX

Security Symposium

San Jose, California, July 1996.

For more information about USENIX Association contact:

1. Phone: (510) 528-8649

2. FAX: (510) 548-5738

3. Email: office@usenix.org

4. WWW URL: http://www.usenix.org

_________________________________________________________________

Secure Deletion of Data from Magnetic and Solid-State Memory

Peter Gutmann

Department of Computer Science

University of Auckland

pgut001@cs.auckland.ac.nz

Abstract

With the use of increasingly sophisticated encryption systems, an

attacker wishing to gain access to sensitive data is forced to look

elsewhere for information. One avenue of attack is the recovery of

supposedly erased data from magnetic media or random-access memory.

This paper covers some of the methods available to recover erased data

and presents schemes to make this recovery significantly more

difficult.

1. Introduction

Much research has gone into the design of highly secure encryption

systems intended to protect sensitive information. However work on

methods of securing (or at least safely deleting) the original

plaintext form of the encrypted data against sophisticated new

analysis techniques seems difficult to find. In the 1980's some work

was done on the recovery of erased data from magnetic media [1] [2]

[3], but to date the main source of information is government

standards covering the destruction of data. There are two main

problems with these official guidelines for sanitizing media. The

first is that they are often somewhat old and may predate newer

techniques for both recording data on the media and for recovering the

recorded data. For example most of the current guidelines on

sanitizing magnetic media predate the early-90's jump in recording

densities, the adoption of sophisticated channel coding techniques

such as PRML, the use of magnetic force microscopy for the analysis of

magnetic media, and recent studies of certain properties of magnetic

media recording such as the behaviour of erase bands. The second

problem with official data destruction standards is that the

information in them may be partially inaccurate in an attempt to fool

opposing intelligence agencies (which is probably why a great many

guidelines on sanitizing media are classified). By deliberately

under-stating the requirements for media sanitization in

publicly-available guides, intelligence agencies can preserve their

information-gathering capabilities while at the same time protecting

their own data using classified techniques.

This paper represents an attempt to analyse the problems inherent in

trying to erase data from magnetic disk media and random-access memory

without access to specialised equipment, and suggests methods for

ensuring that the recovery of data from these media can be made as

difficult as possible for an attacker.

2. Methods of Recovery for Data stored on Magnetic Media

Magnetic force microscopy (MFM) is a recent technique for imaging

magnetization patterns with high resolution and minimal sample

preparation. The technique is derived from scanning probe microscopy

(SPM) and uses a sharp magnetic tip attached to a flexible cantilever

placed close to the surface to be analysed, where it interacts with

the stray field emanating from the sample. An image of the field at

the surface is formed by moving the tip across the surface and

measuring the force (or force gradient) as a function of position. The

strength of the interaction is measured by monitoring the position of

the cantilever using an optical interferometer or tunnelling sensor.

Magnetic force scanning tunneling microscopy (STM) is a more recent

variant of this technique which uses a probe tip typically made by

plating pure nickel onto a prepatterned surface, peeling the resulting

thin film from the substrate it was plated onto and plating it with a

thin layer of gold to minimise corrosion, and mounting it in a probe

where it is placed at some small bias potential (typically a few

tenths of a nanoamp at a few volts DC) so that electrons from the

surface under test can tunnel across the gap to the probe tip (or vice

versa). The probe is scanned across the surface to be analysed as a

feedback system continuously adjusts the vertical position to maintain

a constant current. The image is then generated in the same way as for

MFM [4] [5]. Other techniques which have been used in the past to

analyse magnetic media are the use of ferrofluid in combination with

optical microscopes (which, with gigabit/square inch recording density

is no longer feasible as the magnetic features are smaller than the

wavelength of visible light) and a number of exotic techniques which

require significant sample preparation and expensive equipment. In

comparison, MFM can be performed through the protective overcoat

applied to magnetic media, requires little or no sample preparation,

and can produce results in a very short time.

Even for a relatively inexperienced user the time to start getting

images of the data on a drive platter is about 5 minutes. To start

getting useful images of a particular track requires more than a

passing knowledge of disk formats, but these are well-documented, and

once the correct location on the platter is found a single image would

take approximately 2-10 minutes depending on the skill of the operator

and the resolution required. With one of the more expensive MFM's it

is possible to automate a collection sequence and theoretically

possible to collect an image of the entire disk by changing the MFM

controller software.

There are, from manufacturers sales figures, several thousand SPM's in

use in the field today, some of which have special features for

analysing disk drive platters, such as the vacuum chucks for standard

disk drive platters along with specialised modes of operation for

magnetic media analysis. These SPM's can be used with sophisticated

programmable controllers and analysis software to allow automation of

the data recovery process. If commercially-available SPM's are

considered too expensive, it is possible to build a reasonably capable

SPM for about US$1400, using a PC as a controller [6].

Faced with techniques such as MFM, truly deleting data from magnetic

media is very difficult. The problem lies in the fact that when data

is written to the medium, the write head sets the polarity of most,

but not all, of the magnetic domains. This is partially due to the

inability of the writing device to write in exactly the same location

each time, and partially due to the variations in media sensitivity

and field strength over time and among devices.

In conventional terms, when a one is written to disk the media records

a one, and when a zero is written the media records a zero. However

the actual effect is closer to obtaining a 0.95 when a zero is

overwritten with a one, and a 1.05 when a one is overwritten with a

one. Normal disk circuitry is set up so that both these values are

read as ones, but using specialised circuitry it is possible to work

out what previous "layers" contained. The recovery of at least one or

two layers of overwritten data isn't too hard to perform by reading

the signal from the analog head electronics with a high-quality

digital sampling oscilloscope, downloading the sampled waveform to a

PC, and analysing it in software to recover the previously recorded

signal. What the software does is generate an "ideal" read signal and

subtract it from what was actually read, leaving as the difference the

remnant of the previous signal. Since the analog circuitry in a

commercial hard drive is nowhere near the quality of the circuitry in

the oscilloscope used to sample the signal, the ability exists to

recover a lot of extra information which isn't exploited by the hard

drive electronics (although with newer channel coding techniques such

as PRML (explained further on) which require extensive amounts of

signal processing, the use of simple tools such as an oscilloscope to

directly recover the data is no longer possible).

Using MFM, we can go even further than this. During normal readback, a

conventional head averages the signal over the track, and any remnant

magnetization at the track edges simply contributes a small percentage

of noise to the total signal. The sampling region is too broad to

distinctly detect the remnant magnetization at the track edges, so

that the overwritten data which is still present beside the new data

cannot be recovered without the use of specialised techniques such as

MFM or STM (in fact one of the "official" uses of MFM or STM is to

evaluate the effectiveness of disk drive servo-positioning mechanisms)

[7]. Most drives are capable of microstepping the heads for internal

diagnostic and error recovery purposes (typical error recovery

strategies consist of rereading tracks with slightly changed data

threshold and window offsets and varying the head positioning by a few

percent to either side of the track), but writing to the media while

the head is off-track in order to erase the remnant signal carries too

much risk of making neighbouring tracks unreadable to be useful (for

this reason the microstepping capability is made very difficult to

access by external means).

These specialised techniques also allow data to be recovered from

magnetic media long after the read/write head of the drive is

incapable of reading anything useful. For example one experiment in AC

erasure involved driving the write head with a 40 MHz square wave with

an initial current of 12 mA which was dropped in 2 mA steps to a final

level of 2 mA in successive passes, an order of magnitude more than

the usual write current which ranges from high microamps to low

milliamps. Any remnant bit patterns left by this erasing process were

far too faint to be detected by the read head, but could still be

observed using MFM [8].

Even with a DC erasure process, traces of the previously recorded

signal may persist until the applied DC field is several times the

media coercivity [9].

Deviations in the position of the drive head from the original track

may leave significant portions of the previous data along the track

edge relatively untouched. Newly written data, present as wide

alternating light and dark bands in MFM and STM images, are often

superimposed over previously recorded data which persists at the track

edges. Regions where the old and new data coincide create continuous

magnetization between the two. However, if the new transition is out

of phase with the previous one, a few microns of erase band with no

definite magnetization are created at the juncture of the old and new

tracks. The write field in the erase band is above the coercivity of

the media and would change the magnetization in these areas, but its

magnitude is not high enough to create new well- defined transitions.

One experiment involved writing a fixed pattern of all 1's with a bit

interval of 2.5 m, moving the write head off-track by approximately

half a track width, and then writing the pattern again with a

frequency slightly higher than that of the previously recorded track

for a bit interval of 2.45 m to create all possible phase differences

between the transitions in the old and new tracks. Using a 4.2 m wide

head produced an erase band of approximately 1 m in width when the

old and new tracks were 180 out of phase, dropping to almost nothing

when the two tracks were in-phase. Writing data at a higher frequency

with the original tracks bit interval at 0.5 m and the new tracks bit

interval at 0.49 m allows a single MFM image to contain all possible

phase differences, showing a dramatic increase in the width of the

erase band as the two tracks move from in-phase to 180 out of phase

[10].

In addition, the new track width can exhibit modulation which depends

on the phase relationship between the old and new patterns, allowing

the previous data to be recovered even if the old data patterns

themselves are no longer distinct. The overwrite performance also

depends on the position of the write head relative to the originally

written track. If the head is directly aligned with the track,

overwrite performance is relatively good; as the head moves offtrack,

the performance drops markedly as the remnant components of the

original data are read back along with the newly-written signal. This

effect is less noticeable as the write frequency increases due to the

greater attenuation of the field with distance [11].

When all the above factors are combined it turns out that each track

contains an image of everything ever written to it, but that the

contribution from each "layer" gets progressively smaller the further

back it was made. Intelligence organisations have a lot of expertise

in recovering these palimpsestuous images.

3. Erasure of Data stored on Magnetic Media

The general concept behind an overwriting scheme is to flip each

magnetic domain on the disk back and forth as much as possible (this

is the basic idea behind degaussing) without writing the same pattern

twice in a row. If the data was encoded directly, we could simply

choose the desired overwrite pattern of ones and zeroes and write it

repeatedly. However, disks generally use some form of run-length

limited (RLL) encoding, so that the adjacent ones won't be written.

This encoding is used to ensure that transitions aren't placed too

closely together, or too far apart, which would mean the drive would

lose track of where it was in the data.

To erase magnetic media, we need to overwrite it many times with

alternating patterns in order to expose it to a magnetic field

oscillating fast enough that it does the desired flipping of the

magnetic domains in a reasonable amount of time. Unfortunately, there

is a complication in that we need to saturate the disk surface to the

greatest depth possible, and very high frequency signals only "scratch

the surface" of the magnetic medium. Disk drive manufacturers, in

trying to achieve ever-higher densities, use the highest possible

frequencies, whereas we really require the lowest frequency a disk

drive can produce. Even this is still rather high. The best we can do

is to use the lowest frequency possible for overwrites, to penetrate

as deeply as possible into the recording medium.

The write frequency also determines how effectively previous data can

be overwritten due to the dependence of the field needed to cause

magnetic switching on the length of time the field is applied. Tests

on a number of typical disk drive heads have shown a difference of up

to 20 dB in overwrite performance when data recorded at 40 kFCI (flux

changes per inch), typical of recent disk drives, is overwritten with

a signal varying from 0 to 100 kFCI. The best average performance for

the various heads appears to be with an overwrite signal of around 10

kFCI, with the worst performance being at 100 kFCI [12]. The track

write width is also affected by the write frequency - as the frequency

increases, the write width decreases for both MR and TFI heads. In

[13] there was a decrease in write width of around 20% as the write

frequency was increased from 1 to 40 kFCI, with the decrease being

most marked at the high end of the frequency range. However, the

decrease in write width is balanced by a corresponding increase in the

two side- erase bands so that the sum of the two remains nearly

constant with frequency and equal to the DC erase width for the head.

The media coercivity also affects the width of the write and erase

bands, with their width dropping as the coercivity increases (this is

one of the explanations for the ever-increasing coercivity of newer,

higher-density drives).

To try to write the lowest possible frequency we must determine what

decoded data to write to produce a low-frequency encoded signal.

In order to understand the theory behind the choice of data patterns

to write, it is necessary to take a brief look at the recording

methods used in disk drives. The main limit on recording density is

that as the bit density is increased, the peaks in the analog signal

recorded on the media are read at a rate which may cause them to

appear to overlap, creating intersymbol interference which leads to

data errors. Traditional peak detector read channels try to reduce the

possibility of intersymbol interference by coding data in such a way

that the analog signal peaks are separated as far as possible. The

read circuitry can then accurately detect the peaks (actually the head

itself only detects transitions in magnetisation, so the simplest

recording code uses a transition to encode a 1 and the absence of a

transition to encode a 0. The transition causes a positive/negative

peak in the head output voltage (thus the name "peak detector read

channel"). To recover the data, we differentiate the output and look

for the zero crossings). Since a long string of 0's will make clocking

difficult, we need to set a limit on the maximum consecutive number of

0's. The separation of peaks is implemented as some form of

run-length-limited, or RLL, coding.

The RLL encoding used in most current drives is described by pairs of

run-length limits (d, k), where d is the minimum number of 0 symbols

which must occur between each 1 symbol in the encoded data, and k is

the maximum. The parameters (d, k) are chosen to place adjacent 1's

far enough apart to avoid problems with intersymbol interference, but

not so far apart that we lose synchronisation.

The grandfather of all RLL codes was FM, which wrote one user data bit

followed by one clock bit, so that a 1 bit was encoded as two

transitions (1 wavelength) while a 0 bit was encoded as one transition

( wavelength). A different approach was taken in modified FM (MFM),

which suppresses the clock bit except between adjacent 0's (the

ambiguity in the use of the term MFM is unfortunate. From here on it

will be used to refer to modified FM rather than magnetic force

microscopy). Taking three example sequences 0000, 1111, and 1010,

these will be encoded as 0(1)0(1)0(1)0, 1(0)1(0)1(0)1, and

1(0)0(0)1(0)0 (where the ()s are the clock bits inserted by the

encoding process). The maximum time between 1 bits is now three 0 bits

(so that the peaks are no more than four encoded time periods apart),

and there is always at least one 0 bit (so that the peaks in the

analog signal are at least two encoded time periods apart), resulting

in a (1,3) RLL code. (1,3) RLL/MFM is the oldest code still in general

use today, but is only really used in floppy drives which need to

remain backwards-compatible.

These constraints help avoid intersymbol interference, but the need to

separate the peaks reduces the recording density and therefore the

amount of data which can be stored on a disk. To increase the

recording density, MFM was gradually replaced by (2,7) RLL (the

original "RLL" format), and that in turn by (1,7) RLL, each of which

placed less constraints on the recorded signal.

Using our knowledge of how the data is encoded, we can now choose

which decoded data patterns to write in order to obtain the desired

encoded signal. The three encoding methods described above cover the

vast majority of magnetic disk drives. However, each of these has

several possible variants. With MFM, only one is used with any

frequency, but the newest (1,7) RLL code has at least half a dozen

variants in use. For MFM with at most four bit times between

transitions, the lowest write frequency possible is attained by

writing the repeating decoded data patterns 1010 and 0101. These have

a 1 bit every other "data" bit, and the intervening "clock" bits are

all 0. We would also like patterns with every other clock bit set to 1

and all others set to 0, but these are not possible in the MFM

encoding (such "violations" are used to generate special marks on the

disk to identify sector boundaries). The best we can do here is three

bit times between transitions, which is generated by repeating the

decoded patterns 100100, 010010 and 001001. We should use several

passes with these patterns, as MFM drives are the oldest,

lowest-density drives around (this is especially true for the

very-low-density floppy drives). As such, they are the easiest to

recover data from with modern equipment and we need to take the most

care with them.

From MFM we jump to the next simplest case, which is (1,7) RLL.

Although there can be as many as 8 bit times between transitions, the

lowest sustained frequency we can have in practice is 6 bit times

between transitions. This is a desirable property from the point of

view of the clock-recovery circuitry, and all (1,7) RLL codes seem to

have this property. We now need to find a way to write the desired

pattern without knowing the particular (1,7) RLL code used. We can do

this by looking at the way the drives error-correction system works.

The error- correction is applied to the decoded data, even though

errors generally occur in the encoded data. In order to make this work

well, the data encoding should have limited error amplification, so

that an erroneous encoded bit should affect only a small, finite

number of decoded bits.

Decoded bits therefore depend only on nearby encoded bits, so that a

repeating pattern of encoded bits will correspond to a repeating

pattern of decoded bits. The repeating pattern of encoded bits is 6

bits long. Since the rate of the code is 2/3, this corresponds to a

repeating pattern of 4 decoded bits. There are only 16 possibilities

for this pattern, making it feasible to write all of them during the

erase process. So to achieve good overwriting of (1,7) RLL disks, we

write the patterns 0000, 0001, 0010, 0011, 0100, 0101, 0110, 0111,

1000, 1001, 1010, 1011, 1100, 1101, 1110, and 1111. These patterns

also conveniently cover two of the ones needed for MFM overwrites,

although we should add a few more iterations of the MFM-specific

patterns for the reasons given above.

Finally, we have (2,7) RLL drives. These are similar to MFM in that an

eight-bit-time signal can be written in some phases, but not all. A

six-bit-time signal will fill in the remaining cracks. Using a

encoding rate, an eight-bit-time signal corresponds to a repeating

pattern of 4 data bits. The most common (2,7) RLL code is shown below:

The most common (2,7) RLL Code

Decoded Data (2,7) RLL Encoded Data

00 1000

01 0100

100 001000

101 100100

111 000100

1100 00001000

1101 00100100

The second most common (2,7) RLL code is the same but with the

"decoded data" complemented, which doesn't alter these patterns.

Writing the required encoded data can be achieved for every other

phase using patterns of 0x33, 0x66, 0xCC and 0x99, which are already

written for (1,7) RLL drives.

Six-bit-time patterns can be written using 3-bit repeating patterns.

The all-zero and all-one patterns overlap with the (1,7) RLL patterns,

leaving six others:

001001001001001001001001

2   4   9   2   4   9

in binary or 0x24 0x92 0x49, 0x92 0x49 0x24 and 0x49 0x24 0x92 in hex,

and

011011011011011011011011

6   D   B   6   D   B

in binary or 0x6D 0xB6 0xDB, 0xB6 0xDB 0x6D and 0xDB 0x6D 0xB6 in hex.

The first three are the same as the MFM patterns, so we need only

three extra patterns to cover (2,7) RLL drives.

Although (1,7) is more popular in recent (post-1990) drives, some

older hard drives do still use (2,7) RLL, and with the ever-increasing

reliability of newer drives it is likely that they will remain in use

for some time to come, often being passed down from one machine to

another. The above three patterns also cover any problems with

endianness issues, which weren't a concern in the previous two cases,

but would be in this case (actually, thanks to the strong influence of

IBM mainframe drives, everything seems to be uniformly big-endian

within bytes, with the most significant bit being written to the disk

first).

The latest high-density drives use methods like Partial-Response

Maximum-Likelihood (PRML) encoding, which may be roughly equated to

the trellis encoding done by V.32 modems in that it is effective but

computationally expensive. PRML codes are still RLL codes, but with

somewhat different constraints. A typical code might have (0,4,4)

constraints in which the 0 means that 1's in a data stream can occur

right next to 0's (so that peaks in the analog readback signal are not

separated), the first 4 means that there can be no more than four 0's

between 1's in a data stream, and the second 4 specifies the maximum

number of 0's between 1's in certain symbol subsequences. PRML codes

avoid intersymbol influence errors by using digital filtering

techniques to shape the read signal to exhibit desired frequency and

timing characteristics (this is the "partial response" part of PRML)

followed by maximum- likelihood digital data detection to determine

the most likely sequence of data bits that was written to the disk

(this is the "maximum likelihood" part of PRML). PRML channels achieve

the same low bit error rate as standard peak-detection methods, but

with much higher recording densities, while using the same heads and

media. Several manufacturers are currently engaged in moving their

peak-detection-based product lines across to PRML, giving a 30-40%

density increase over standard RLL channels [14].

Since PRML codes don't try to separate peaks in the same way that

non-PRML RLL codes do, all we can do is to write a variety of random

patterns because the processing inside the drive is too complex to

second- guess. Fortunately, these drives push the limits of the

magnetic media much more than older drives ever did by encoding data

with much smaller magnetic domains, closer to the physical capacity of

the magnetic media (the current state of the art in PRML drives has a

track density of around 6700 TPI (tracks per inch) and a data

recording density of 170 kFCI, nearly double that of the nearest (1,7)

RLL equivalent. A convenient side-effect of these very high recording

densities is that a written transition may experience the write field

cycles for successive transitions, especially at the track edges where

the field distribution is much broader [15]. Since this is also where

remnant data is most likely to be found, this can only help in

reducing the recoverability of the data). If these drives require

sophisticated signal processing just to read the most recently written

data, reading overwritten layers is also correspondingly more

difficult. A good scrubbing with random data will do about as well as

can be expected.

We now have a set of 22 overwrite patterns which should erase

everything, regardless of the raw encoding. The basic disk eraser can

be improved slightly by adding random passes before and after the

erase process, and by performing the deterministic passes in random

order to make it more difficult to guess which of the known data

passes were made at which point. To deal with all this in the

overwrite process, we use the sequence of 35 consecutive writes shown

below:

Overwrite Data

Pass No.    Data Written                            Encoding Scheme Targeted

1        Random

2        Random

3        Random

4        Random

5        01010101 01010101 01010101 0x55              (1,7) RLL MFM

6        10101010 10101010 10101010 0xAA              (1,7) RLL MFM

7        10010010 01001001 00100100 0x92 0x49 0x24    (2,7) RLL MFM

8        01001001 00100100 10010010 0x49 0x24 0x92    (2,7) RLL MFM

9        00100100 10010010 01001001 0x24 0x92 0x49    (2,7) RLL MFM

10        00000000 00000000 00000000 0x00              (1,7) RLL (2,7) RLL

11        00010001 00010001 00010001 0x11              (1,7) RLL

12        00100010 00100010 00100010 0x22              (1,7) RLL

13        00110011 00110011 00110011 0x33              (1,7) RLL (2,7) RLL

14        01000100 01000100 01000100 0x44              (1,7) RLL

15        01010101 01010101 01010101 0x55              (1,7) RLL MFM

16        01100110 01100110 01100110 0x66              (1,7) RLL (2,7) RLL

17        01110111 01110111 01110111 0x77              (1,7) RLL

18        10001000 10001000 10001000 0x88              (1,7) RLL

19        10011001 10011001 10011001 0x99              (1,7) RLL (2,7) RLL

20        10101010 10101010 10101010 0xAA              (1,7) RLL MFM

21        10111011 10111011 10111011 0xBB              (1,7) RLL

22        11001100 11001100 11001100 0xCC              (1,7) RLL (2,7) RLL

23        11011101 11011101 11011101 0xDD              (1,7) RLL

24        11101110 11101110 11101110 0xEE              (1,7) RLL

25        11111111 11111111 11111111 0xFF              (1,7) RLL (2,7) RLL

26        10010010 01001001 00100100 0x92 0x49 0x24    (2,7) RLL MFM

27        01001001 00100100 10010010 0x49 0x24 0x92    (2,7) RLL MFM

28        00100100 10010010 01001001 0x24 0x92 0x49    (2,7) RLL MFM

29        01101101 10110110 11011011 0x6D 0xB6 0xDB    (2,7) RLL

30        10110110 11011011 01101101 0xB6 0xDB 0x6D    (2,7) RLL

31        11011011 01101101 10110110 0xDB 0x6D 0xB6    (2,7) RLL

32        Random

33        Random

34        Random

35        Random

The MFM-specific patterns are repeated twice because MFM drives have

the lowest density and are thus particularly easy to examine. The

deterministic patterns between the random writes are permuted before

the write is performed, to make it more difficult for an opponent to

use knowledge of the erasure data written to attempt to recover

overwritten data (in fact we need to use a cryptographically strong

random number generator to perform the permutations to avoid the

problem of an opponent who can read the last overwrite pass being able

to predict the previous passes and "echo cancel" passes by subtracting

the known overwrite data).

If the device being written to supports caching or buffering of data,

this should be disabled to ensure that physical disk writes are

performed for each pass instead of everything but the last pass being

lost in the buffering. For example physical disk access can be forced

during SCSI-2 Group 1 write commands by setting the Force Unit Access

bit in the SCSI command block (although at least one popular drive has

a bug which causes all writes to be ignored when this bit is set -

remember to test your overwrite scheme before you deploy it). Another

consideration which needs to be taken into account when trying to

erase data through software is that drives conforming to some of the

higher-level protocols such as the various SCSI standards are

relatively free to interpret commands sent to them in whichever way

they choose (as long as they still conform to the SCSI specification).

Thus some drives, if sent a FORMAT UNIT command may return immediately

without performing any action, may simply perform a read test on the

entire disk (the most common option), or may actually write data to

the disk (the SCSI- 2 standard includes an initialization pattern (IP)

option for the FORMAT UNIT command, however this is not necessarily

supported by existing drives).

If the data is very sensitive and is stored on floppy disk, it can

best be destroyed by removing the media from the disk liner and

burning it, or by burning the entire disk, liner and all (most floppy

disks burn remarkably well - albeit with quantities of oily smoke -

and leave very little residue).

4. Other Methods of Erasing Magnetic Media

The previous section has concentrated on erasure methods which require

no specialised equipment to perform the erasure. Alternative means of

erasing media which do require specialised equipment are degaussing (a

process in which the recording media is returned to its initial state)

and physical destruction. Degaussing is a reasonably effective means

of purging data from magnetic disk media, and will even work through

most drive cases (research has shown that the aluminium housings of

most disk drives attenuate the degaussing field by only about 2 dB

[16]).

The switching of a single-domain magnetic particle from one

magnetization direction to another requires the overcoming of an

energy barrier, with an external magnetic field helping to lower this

barrier. The switching depends not only on the magnitude of the

external field, but also on the length of time for which it is

applied. For typical disk drive media, the short-term field needed to

flip enough of the magnetic domains to be useful in recording a signal

is about 1/3 higher than the coercivity of the media (the exact figure

varies with different media types) [17].

However, to effectively erase a medium to the extent that recovery of

data from it becomes uneconomical requires a magnetic force of about

five times the coercivity of the medium [18], although even small

external magnetic fields are sufficient to upset the normal operation

of a hard disk (typically a few gauss at DC, dropping to a few

milligauss at 1 MHz). Coercivity (measured in Oersteds, Oe) is a

property of magnetic material and is defined as the amount of magnetic

field necessary to reduce the magnetic induction in the material to

zero - the higher the coercivity, the harder it is to erase data from

a medium. Typical figures for various types of magnetic media are

given below:

Typical Media Coercivity Figures

Medium Coercivity

5.25" 360K floppy disk 300 Oe

5.25" 1.2M floppy disk 675 Oe

3.5" 720K floppy disk 300 Oe

3.5" 1.44M floppy disk 700 Oe

3.5" 2.88M floppy disk 750 Oe

3.5" 21M floptical disk 750 Oe

Older (1980's) hard disks 900-1400 Oe

Newer (1990's) hard disks 1400-2200 Oe

1/2" magnetic tape 300 Oe

1/4" QIC tape 550 Oe

8 mm metallic particle tape 1500 Oe

DAT metallic particle tape 1500 Oe

US Government guidelines class tapes of 350 Oe coercivity or less as

low-energy or Class I tapes and tapes of 350-750 Oe coercivity as

high-energy or Class II tapes. Degaussers are available for both types

of tapes. Tapes of over 750 Oe coercivity are referred to as Class

III, with no known degaussers capable of fully erasing them being

known [19], since even the most powerful commercial AC degausser

cannot generate the recommended 7,500 Oe needed for full erasure of a

typical DAT tape currently used for data backups.

Degaussing of disk media is somewhat more difficult - even older hard

disks generally have a coercivity equivalent to Class III tapes,

making them fairly difficult to erase at the outset. Since

manufacturers rate their degaussers in peak gauss and measure the

field at a certain orientation which may not be correct for the type

of medium being erased, and since degaussers tend to be rated by

whether they erase sufficiently for clean rerecording rather than

whether they make the information impossible to recover, it may be

necessary to resort to physical destruction of the media to completely

sanitise it (in fact since degaussing destroys the sync bytes, ID

fields, error correction information, and other paraphernalia needed

to identify sectors on the media, thus rendering the drive unusable,

it makes the degaussing process mostly equivalent to physical

destruction). In addition, like physical destruction, it requires

highly specialised equipment which is expensive and difficult to

obtain (one example of an adequate degausser was the 2.5 MW Navy

research magnet used by a former Pentagon site manager to degauss a

14" hard drive for 1 minutes. It bent the platters on the drive and

probably succeeded in erasing it beyond the capabilities of any data

recovery attempts [20]).

5. Further Problems with Magnetic Media

A major issue which cannot be easily addressed using any standard

software-based overwrite technique is the problem of defective sector

handling. When the drive is manufactured, the surface is scanned for

defects which are added to a defect list or flaw map. If further

defects, called grown defects, occur during the life of the drive,

they are added to the defect list by the drive or by drive management

software. There are several techniques which are used to mask the

defects in the defect list. The first, alternate tracks, moves data

from tracks with defects to known good tracks. This scheme is the

simplest, but carries a high access cost, as each read from a track

with defects requires seeking to the alternate track and a rotational

latency delay while waiting for the data location to appear under the

head, performing the read or write, and, if the transfer is to

continue onto a neighbouring track, seeking back to the original

position. Alternate tracks may be interspersed among data tracks to

minimise the seek time to access them.

A second technique, alternate sectors, allocates alternate sectors at

the end of the track to minimise seeks caused by defective sectors.

This eliminates the seek delay, but still carries some overhead due to

rotational latency. In addition it reduces the usable storage capacity

by 1-3%.

A third technique, inline sector sparing, again allocates a spare

sector at the end of each track, but resequences the sector ID's to

skip the defective sector and include the spare sector at the end of

the track, in effect pushing the sectors past the defective one

towards the end of the track. The associated cost is the lowest of the

three, being one sector time to skip the defective sector [21].

The handling of mapped-out sectors and tracks is an issue which can't

be easily resolved without the cooperation of hard drive

manufacturers. Although some SCSI and IDE hard drives may allow access

to defect lists and even to mapped-out areas, this must be done in a

highly manufacturer- and drive-specific manner. For example the SCSI-2

READ DEFECT DATA command can be used to obtain a list of all defective

areas on the drive. Since SCSI logical block numbers may be mapped to

arbitrary locations on the disk, the defect list is recorded in terms

of heads, tracks, and sectors. As all SCSI device addressing is

performed in terms of logical block numbers, mapped-out sectors or

tracks cannot be addressed. The only reasonably portable possibility

is to clear various automatic correction flags in the read-write error

recovery mode page to force the SCSI device to report read/write

errors to the user instead of transparently remapping the defective

areas. The user can then use the READ LONG and WRITE LONG commands

(which allow access to sectors and extra data even in the presence of

read/write errors), to perform any necessary operations on the

defective areas, and then use the REASSIGN BLOCKS command to reassign

the defective sections. However this operation requires an in-depth

knowledge of the operation of the SCSI device and extensive changes to

disk drivers, and more or less defeats the purpose of having an

intelligent peripheral.

The ANSI X3T-10 and X3T-13 subcommittees are currently looking at

creating new standards for a Universal Security Reformat command for

IDE and SCSI hard disks which will address these issues. This will

involve a multiple-pass overwrite process which covers mapped-out disk

areas with deliberate off-track writing. Many drives available today

can be modified for secure erasure through a firmware upgrade, and

once the new firmware is in place the erase procedure is handled by

the drive itself, making unnecessary any interaction with the host

system beyond the sending of the command which begins the erase

process.

Long-term ageing can also have a marked effect on the erasability of

magnetic media. For example, some types of magnetic tape become

increasingly difficult to erase after being stored at an elevated

temperature or having contained the same magnetization pattern for a

considerable period of time [22]. The same applies for magnetic disk

media, with decreases in erasability of several dB being recorded

[23]. The erasability of the data depends on the amount of time it has

been stored on the media, not on the age of the media itself (so that,

for example, a five-year-old freshly-written disk is no less erasable

than a new freshly-written disk).

The dependence of media coercivity on temperature can affect overwrite

capability if the data was initially recorded at a temperature where

the coercivity was low (so that the recorded pattern penetrated deep

into the media), but must be overwritten at a temperature where the

coercivity is relatively high. This is important in hard disk drives,

where the temperature varies depending on how long the unit has been

used and, in the case of drives with power-saving features enabled,

how recently and frequently it has been used. However the overwrite

performance depends not only on temperature-dependent changes in the

media, but also on temperature-dependent changes in the read/write

head. Thankfully the combination of the most common media used in

current drives with various common types of read/write heads produce a

change in overwrite performance of only a few hundredths of a decibel

per degree over the temperature range -40C to + 40C, as changes in

the head compensate for changes in the media [24].

Another issue which needs to be taken into account is the ability of

most newer storage devices to recover from having a remarkable amount

of damage inflicted on them through the use of various

error-correction schemes. As increasing storage densities began to

lead to multiple-bit errors, manufacturers started using sophisticated

error-correction codes (ECC's) capable of correcting multiple error

bursts. A typical drive might have 512 bytes of data, 4 bytes of CRC,

and 11 bytes of ECC per sector. This ECC would be capable of

correcting single burst errors of up to 22 bits or double burst errors

of up to 11 bits, and can detect a single burst error of up to 51 bits

or three burst errors of up to 11 bits in length [25]. Another drive

manufacturer quotes the ability to correct up to 120 bits, or up to 32

bits on the fly, using 198-bit Reed-Solomon ECC [26]. Therefore even

if some data is reliably erased, it may be possible to recover it

using the built-in error-correction capabilities of the drive.

Conversely, any erasure scheme which manages to destroy the ECC

information (for example through the use of the SCSI-2 WRITE LONG

command which can be used to write to areas of a disk sector outside

the normal data areas) stands a greater chance of making the data

unrecoverable.

6. Sidestepping the Problem

The easiest way to solve the problem of erasing sensitive information

from magnetic media is to ensure that it never gets to the media in

the first place. Although not practical for general data, it is often

worthwhile to take steps to keep particularly important information

such as encryption keys from ever being written to disk. This would

typically happen when the memory containing the keys is paged out to

disk by the operating system, where they can then be recovered at a

later date, either manually or using software which is aware of the

in-memory data format and can locate it automatically in the swap file

(for example there exists software which will search the Windows swap

file for keys from certain DOS encryption programs). An even worse

situation occurs when the data is paged over a network, allowing

anyone with a packet sniffer or similar tool on the same subnet to

observe the information (for example there exists software which will

monitor and even alter NFS traffic on the fly which could be modified

to look for known in-memory data patterns moving to and from a

networked swap disk [27]).

To solve these problems the memory pages containing the information

can be locked to prevent them from being paged to disk or transmitted

over a network. This approach is taken by at least one encryption

library, which allocates all keying information inside protected

memory blocks visible to the user only as opaque handles, and then

optionally locks the memory (provided the underlying OS allows it) to

prevent it from being paged [28]. The exact details of locking pages

in memory depend on the operating system being used. Many Unix systems

now support the mlock()/munlock() calls or have some alternative

mechanism hidden among the mmap()-related functions which can be used

to lock pages in memory. Unfortunately these operations require

superuser priviledges because of their potential impact on system

performance if large ranges of memory are locked. Other systems such

as Microsoft Windows NT allow user processes to lock memory with the

VirtualLock()/VirtualUnlock() calls, but limit the total number of

regions which can be locked.

Most paging algorithms are relatively insensitive to having sections

of memory locked, and can even relocate the locked pages (since the

logical to physical mapping is invisible to the user), or can move the

pages to a "safe" location when the memory is first locked. The main

effect of locking pages in memory is to increase the minimum working

set size which, taken in moderation, has little noticeable effect on

performance. The overall effects depend on the operating system and/or

hardware implementations of virtual memory. Most Unix systems have a

global page replacement policy in which a page fault may be satisfied

by any page frame. A smaller number of operating systems use a local

page replacement policy in which pages are allocated from a fixed (or

occasionally dynamically variable) number of page frames allocated on

a per- process basis. This makes them much more sensitive to the

effects of locking pages, since every locked page decreases the

(finite) number of pages available to the process. On the other hand

it makes the system as a whole less sensitive to the effects of one

process locking a large number of pages. The main effective difference

between the two is that under a local replacement policy a process can

only lock a small fixed number of pages without affecting other

processes, whereas under a global replacement policy the number of

pages a process can lock is determined on a system-wide basis and may

be affected by other processes.

In practice neither of these allocation strategies seem to cause any

real problems. Although any practical measurements are very difficult

to perform since they vary wildly depending on the amount of physical

memory present, paging strategy, operating system, and system load, in

practice locking a dozen 1K regions of memory (which might be typical

of a system on which a number of users are running programs such as

mail encryption software) produced no noticeable performance

degradation observable by system- monitoring tools. On machines such

as network servers handling large numbers of secure connections (for

example an HTTP server using SSL), the effects of locking large

numbers of pages may be more noticeable.

7. Methods of Recovery for Data stored in Random-Access Memory

Contrary to conventional wisdom, "volatile" semiconductor memory does

not entirely lose its contents when power is removed. Both static

(SRAM) and dynamic (DRAM) memory retains some information on the data

stored in it while power was still applied. SRAM is particularly

susceptible to this problem, as storing the same data in it over a

long period of time has the effect of altering the preferred power-up

state to the state which was stored when power was removed. Older SRAM

chips could often "remember" the previously held state for several

days. In fact, it is possible to manufacture SRAM's which always have

a certain state on power-up, but which can be overwritten later on - a

kind of "writeable ROM".

DRAM can also "remember" the last stored state, but in a slightly

different way. It isn't so much that the charge (in the sense of a

voltage appearing across a capacitance) is retained by the RAM cells,

but that the thin oxide which forms the storage capacitor dielectric

is highly stressed by the applied field, or is not stressed by the

field, so that the properties of the oxide change slightly depending

on the state of the data. One thing that can cause a threshold shift

in the RAM cells is ionic contamination of the cell(s) of interest,

although such contamination is rarer now than it used to be because of

robotic handling of the materials and because the purity of the

chemicals used is greatly improved. However, even a perfect oxide is

subject to having its properties changed by an applied field. When it

comes to contaminants, sodium is the most common offender - it is

found virtually everywhere, and is a fairly small (and therefore

mobile) atom with a positive charge. In the presence of an electric

field, it migrates towards the negative pole with a velocity which

depends on temperature, the concentration of the sodium, the oxide

quality, and the other impurities in the oxide such as dopants from

the processing. If the electric field is zero and given enough time,

this stress tends to dissipate eventually.

The stress on the cell is a cumulative effect, much like charging an

RC circuit. If the data is applied for only a few milliseconds then

there is very little "learning" of the cell, but if it is applied for

hours then the cell will acquire a strong (relatively speaking) change

in its threshold. The effects of the stress on the RAM cells can be

measured using the built-in self test capabilities of the cells, which

provide the ability to impress a weak voltage on a storage cell in

order to measure its margin. Cells will show different margins

depending on how much oxide stress has been present. Many DRAM's have

undocumented test modes which allow some normal I/O pin to become the

power supply for the RAM core when the special mode is active. These

test modes are typically activated by running the RAM in a nonstandard

configuration, so that a certain set of states which would not occur

in a normally-functioning system has to be traversed to activate the

mode. Manufacturers won't admit to such capabilities in their products

because they don't want their customers using them and potentially

rejecting devices which comply with their spec sheets, but have little

margin beyond that.

A simple but somewhat destructive method to speed up the annihilation

of stored bits in semiconductor memory is to heat it. Both DRAM's and

SRAM's will lose their contents a lot more quickly at Tjunction =

140C than they will at room temperature. Several hours at this

temperature with no power applied will clear their contents

sufficiently to make recovery difficult. Conversely, to extend the

life of stored bits with the power removed, the temperature should be

dropped below -60C. Such cooling should lead to weeks, instead of

hours or days, of data retention.

8. Erasure of Data stored in Random-Access Memory

Simply repeatedly overwriting the data held in DRAM with new data

isn't nearly as effective as it is for magnetic media. The new data

will begin stressing or relaxing the oxide as soon as it is written,

and the oxide will immediately begin to take a "set" which will either

reinforce the previous "set" or will weaken it. The greater the amount

of time that new data has existed in the cell, the more the old stress

is "diluted", and the less reliable the information extraction will

be. Generally, the rates of change due to stress and relaxation are in

the same order of magnitude. Thus, a few microseconds of storing the

opposite data to the currently stored value will have little effect on

the oxide. Ideally, the oxide should be exposed to as much stress at

the highest feasible temperature and for as long as possible to get

the greatest "erasure" of the data. Unfortunately if carried too far

this has a rather detrimental effect on the life expectancy of the

RAM.

Therefore the goal to aim for when sanitising memory is to store the

data for as long as possible rather than trying to change it as often

as possible. Conversely, storing the data for as short a time as

possible will reduce the chances of it being "remembered" by the cell.

Based on tests on DRAM cells, a storage time of one second causes such

a small change in threshold that it probably isn't detectable. On the

other hand, one minute is probably detectable, and 10 minutes is

certainly detectable.

The most practical solution to the problem of DRAM data retention is

therefore to constantly flip the bits in memory to ensure that a

memory cell never holds a charge long enough for it to be

"remembered". While not practical for general use, it is possible to

do this for small amounts of very sensitive data such as encryption

keys. This is particularly advisable where keys are stored in the same

memory location for long periods of time and control access to large

amounts of information, such as keys used for transparent encryption

of files on disk drives. The bit-flipping also has the convenient

side-effect of keeping the page containing the encryption keys at the

top of the queue maintained by the system's paging mechanism, greatly

reducing the chances of it being paged to disk at some point.

9. Conclusion

Data overwritten once or twice may be recovered by subtracting what is

expected to be read from a storage location from what is actually

read. Data which is overwritten an arbitrarily large number of times

can still be recovered provided that the new data isn't written to the

same location as the original data (for magnetic media), or that the

recovery attempt is carried out fairly soon after the new data was

written (for RAM). For this reason it is effectively impossible to

sanitise storage locations by simple overwriting them, no matter how

many overwrite passes are made or what data patterns are written.

However by using the relatively simple methods presented in this paper

the task of an attacker can be made significantly more difficult, if

not prohibitively expensive.

Acknowledgments

The author would like to thank Nigel Bree, Peter Fenwick, Andy

Hospodor, Kevin Martinez, Colin Plumb, and Charles Preston for their

advice and input during the preparation of this paper.

References

[1] "Emergency Destruction of Information Storing Media", M.Slusarczuk

et al, Institute for Defense Analyses, December 1987.

[2] "A Guide to Understanding Data Remanence in Automated Information

Systems", National Computer Security Centre, September 1991.

[3] "Detection of Digital Information from Erased Magnetic Disks",

Venugopal Veeravalli, Masters thesis, Carnegie-Mellon University,

1987.

[4] "Magnetic force microscopy: General principles and application to

longitudinal recording media", D.Rugar, H.Mamin, P.Guenther,

S.Lambert, J.Stern, I.McFadyen, and T.Yogi, Journal of Applied

Physics, Vol.68, No.3 (August 1990), p.1169.

[5] "Tunneling-stabilized Magnetic Force Microscopy of Bit Tracks on a

Hard Disk", Paul Rice and John Moreland, IEEE Trans.on Magnetics,

Vol.27, No.3 (May 1991), p.3452.

[6] "NanoTools: The Homebrew STM Page", Jim Rice, NanoTools: The

Homebrew STM Page.

[7] "Magnetic Force Scanning Tunnelling Microscope Imaging of

Overwritten Data", Romel Gomez, Amr Adly, Isaak Mayergoyz, Edward

Burke, IEEE Trans.on Magnetics, Vol.28, No.5 (September 1992), p.3141.

[8] "Comparison of Magnetic Fields of Thin-Film Heads and Their

Corresponding Patterns Using Magnetic Force Microscopy", Paul Rice,

Bill Hallett, and John Moreland, IEEE Trans.on Magnetics, Vol.30, No.6

(November 1994), p.4248.

[9] "Computation of Magnetic Fields in Hysteretic Media", Amr Adly,

Isaak Mayergoyz, Edward Burke, IEEE Trans.on Magnetics, Vol.29, No.6

(November 1993), p.2380.

[10] "Magnetic Force Microscopy Study of Edge Overwrite

Characteristics in Thin Film Media", Jian- Gang Zhu, Yansheng Luo, and

Juren Ding, IEEE Trans.on Magnetics, Vol.30, No.6 (November 1994),

p.4242.

[11] "Microscopic Investigations of Overwritten Data", Romel Gomez,

Edward Burke, Amr Adly, Isaak Mayergoyz, J.Gorczyca, Journal of

Applied Physics, Vol.73, No.10 (May 1993), p.6001.

[12] "Relationship between Overwrite and Transition Shift in

Perpendicular Magnetic Recording", Hiroaki Muraoka, Satoshi Ohki, and

Yoshihisa Nakamura, IEEE Trans.on Magnetics, Vol.30, No.6 (November

1994), p.4272.

[13] "Effects of Current and Frequency on Write, Read, and Erase

Widths for Thin-Film Inductive and Magnetoresistive Heads", Tsann Lin,

Jodie Christner, Terry Mitchell, Jing-Sheng Gau, and Peter George,

IEEE Trans.on Magnetics, Vol.25, No.1 (January 1989), p.710.

[14] "PRML Read Channels: Bringing Higher Densities and Performance to

New-Generation Hard Drives", Quantum Corporation, 1995.

[15] "Density and Phase Dependence of Edge Erase Band in MR/Thin Film

Head Recording", Yansheng Luo, Terence Lam, Jian-Gang Zhu, IEEE

Trans.on Magnetics, Vol.31, No.6 (November 1995), p.3105.

[16] "A Guide to Understanding Data Remanence in Automated Information

Systems", National Computer Security Centre, September 1991.

[17] "Time-dependant Magnetic Phenomena and Particle-size Effects in

Recording Media", IEEE Trans.on Magnetics, Vol.26, No.1 (January

1990), p.193.

[18] "The Data Dilemna", Charles Preston, Security Management Journal,

February 1995.

[19] "Magnetic Tape Degausser", NSA/CSS Specification L14-4-A, 31

October 1985.

[20] "How many times erased does DoD want?", David Hayes, posting to

comp.periphs.scsi newsgroup, 24 July 1991, message-ID

1991Jul24.050701.16005@sulaco.lone star.org.

[21] "The Changing Nature of Disk Controllers", Andrew Hospodor and

Albert Hoagland, Proceedings of the IEEE, Vol.81, No.4 (April 1993),

p.586.

[22] "Annealing Study of the Erasability of High Energy Tapes",

L.Lekawat, G.Spratt, and M.Kryder, IEEE Trans.on Magnetics, Vol.29,

No.6 (November 1993), p.3628.

[23] "The Effect of Aging on Erasure in Particulate Disk Media",

K.Mountfield and M.Kryder, IEEE Trans.on Magnetics, Vol.25, No 5

(September 1989), p.3638.

[24] "Overwrite Temperature Dependence for Magnetic Recording",

Takayuki Takeda, Katsumichi Tagami, and Takaaki Watanabe, Journal of

Applied Physics, Vol.63, No.8 (April 1988), p.3438.

[25] Conner 3.5" hard drive data sheets, 1994, 1995.

[26] "Technology and Time-to-Market: The Two Go Hand-in-Hand", Quantum

Corporation, 1995.

[27] "Basic Flaws in Internet Security and Commerce", Paul Gauthier,

posting to comp.security.unix newsgroup, 9 October 1995, message-ID

gauthier.813274073@espresso.cs.ber keley.edu.

[28] "cryptlib Free Encryption Library", Peter Gutmann, cryptlib.

_________________________________________________________________

Secure Deletion of Data from Magnetic and Solid-State Memory / Peter

Gutmann / pgut001@cs.auckland.ac.nz