P80BOX Documentation by The Researcher June 3, 1994

INTRODUCTION

The signaling tones produced by a Blue Box, Red Box, Green box, Silver Box

and a standard Touch Tone pad are generated by this program with laboratory

precision. They are provided, together with this file, as historical

documentation of the golden age of hardware phone phreaking.

P80BOX programs the FM synthesizer chips on an Adlib or Sound Blaster card

to produce multiple sine wave outputs. Its predecessors were BLSTRBOX and

DACBOX which use a different method for producing sound. BLSTRBOX uses the

digital-to-analog converter on a Sound Blaster card. DACBOX uses a D to A

convertor attached to a printer port.

Information for P80BOX and this documentation came from many sources.

These are the main ones in order of proven usefulness:

1. The Whistler, most knowledgeable phone phreak of all time.

2. Bell Operating Company documents.

3. The computer underground (contacted through P-80 Systems).

4. Articles published in *TAP, Esquire and 73 Magazine.

5. University libraries.

* TAP was an underground newsletter that appeared in 1971 before the age of

the personal computer. It specialized in general anarchy and serious phone

phreaking.

Except for the Green Box and the European break signal, I have personally

tested and proven everything discussed in this file by field research conducted

in the mid 80's using less sophisticated technology than that provided by this

program. The European break signal appeared in a number of Blue Box programs

from European countries so I have provided support for it. I rely on the

reports of European programmers for belief in its validity.

QUICK START

At the dos prompt type: P80BOX <ENTER>

Or to force monochrome mode: P80BOX /M <ENTER>

The program will come up in MF mode. In this mode the Blue Box, Red Box

and Green Box keys are active. For help on accessing other modes and features,

press F1.

RUNNING P80BOX

HARDWARE REQUIREMENTS: 1. An IBM or compatible personal computer.

2. Any type of monitor. Monochrome and VGA have been

tested. Only ASCII graphics are used requiring no

graphics modes. Monitor type will be autodetected.

Monochrome mode can be forced by using the /M switch.

3. An Adlib or Sound Blaster card.

The program will come up in MF mode. MF is an abbreviation of

multifrequency. These are the Blue Box tones. DTMF tones (Touch Tones) are also

produced from this screen by enclosing the digits in square brackets "[]".

Tones supplied by the user are played by enclosing the digits in curly braces

"{}". User tones are loaded from an external file provided by the user. Use the

file TEMPLET.TBL as a guide to setting up your own user table.

The frequencies and tolerances for all the tones used are given in the

section on TECHNICAL SPECIFICATIONS.

BLUE BOX KEYS: 1, 2, 3, 4, 5, 6, 7, 8, 9, 0

E = Code 11

T = Code 12

K = KP (Key Pulses)

P = KP2

S = ST

D = 2600 Hz Disconnect

B = European Break - Appears to be used like our 2600 Hz

disconnect.

RED BOX KEYS:  Q = Quarter

I = dIme

N = Nickel

GREEN BOX KEYS: R - Coin Return

O - Coin Collect

G - Ringback

DTMF KEYS:    1, 2, 3, 4, 5, 6, 7, 8, 9, 0, *, #, A, B, C, D

SILVER BOX:   These are the DTMF A-D keys as follows:

A = Flash

B = Flash Override Priority

C = Priority Communication

D = Priority Override

DELAYS:       ,  = (comma) 1 second delay

~  = (tilde) 10 second delay

Mx = Delay x milliseconds. x is in the range of

1 - 65535. Leave a space after the last digit in

milliseconds.

W  = Wait. Suspend dialing until a key is pressed.

L  = Long play. The next digit is turned on and left on

until a key is pressed.

OTHER KEYS:   X - Switch to manual dialing

? - .5 seconds of 480Hz. To mark tape position.

[ = Turn on DTMF interpretation

] = Turn off DTMF interpretation

{ = Turn on USER interpretation

} = Turn off USER interpretation

PLAYING DTMF TONES: Enclose the digits to play as standard Touch Tones in

square brackets. For example, to dial 555-1234, type [555-1234] and hit

<ENTER>. When the program encounters a left square bracket '[' DTMF

interpretation is turned on. It stays on for as long as the program runs until

a right square bracket ']' is encountered. This feature is handy if you are

doing an experiment that only requires DTMF tones. You can start the first dial

string with a left square bracket leaving off the right bracket or just type a

left bracket by itself and hit <ENTER>. All subsequent dial strings will be

interpreted as DTMF digits without the need for brackets. You can switch back

to Blue Box/Red Box mode at any time by entering a right square bracket

anywhere in a dial string or by itself and hitting <ENTER>. A message will

appear on the main screen when the program has been left in DTMF mode.

All you have to remember about square brackets and DTMF tones is:

[ = Turn on DTMF interpretation

] = Turn off DTMF interpretation

PLAYING USER TONES: Playing user tones works exactly like playing DTMF tones

except digits are enclosed in curly braces instead of square brackets. For

example, to dial 555-1234, type {555-1234} <ENTER>. User tones are taken from a

table of user defined frequencies contained in an external file. The name of

the file containing the user table can be added to the command line when P80BOX

is run or can be loaded later by pressing F3. Use the file TEMPLET.TBL as a

guide to setting up your own user table.

Example of loading user table from command line: P80BOX user.tbl

THE FUNCTION KEYS

Pressing F1 displays the following help screen:

����������[FUNCTION KEYS]�������������������������[DIALING MODES]���������������

F1  - Display this help screen     �  [] - Digits enclosed in square brackets

F2  - Setup dialing speed          �       are played as DTMF tones. These are

F3  - Load user table              �       standard Touch-Tones.

F4  - Manual dialing (toggle)      �  {} - Digits enclosed in curly brackets

F5  - Ear training (phone sounds)  �       are taken from the user table of

F6  - Load dial string from disk   �       frequencies. These must be loaded

F7  - Save dial string to disk     �       from an external file created by the

F8  - Rotary bluebox (toggle)      �       user. The filename can be entered on

F9  - Setup user key               �       the command line or the file can be

F10 - Recall last dial string      �       loaded by pressing F3.

��������������[DELAYS]������������Ĵ

,  = (comma) 1 second delay        �  When a mode is turned on, it stays on

~  = (tilde) 10 second delay       �  until its corresponding right bracket is

Mx = Delay x milliseconds.         �  encountered. The modes are MF, DTMF,

x is in the range of          �  USER and ROTARY BLUEBOX. The default mode

1 - 65535. Leave a space      �  is MF. In this mode the blue, red and

after the last digit in       �  green box tones are available. Silver box

milliseconds.                 �  (also know as white box) tones and Touch

W  = Wait. Suspend dialing         �  Tones are available in DTMF mode.

until a key is pressed.       �

L  = Long play. The next digit     ������������[SPECIAL PURPOSE KEYS]�����������

is turned on and left on      �  X - Switch to manual dialing

until a key is pressed.       �  ? - .5 seconds of 480Hz. To mark tape pos.

��������������������������������������������������������������������������������

F2 - Setup dialing speed

This lets you change the duration and interdigit interval of the MF (Blue

Box) and DTMF (Touch-Tone) digits. ST is included as an MF digit. KP1, KP2 and

2600Hz disconnect are set separately. You are given the option of making the

new settings permanent defaults for the program.

F3 - Load user table

Prompts for the name of the file containing the table of user defined

frequencies.

F4 - Manual dialing (toggle)

When manual dialing is active digits are played immediately when pressed.

F5 - Ear training (phone sounds)

This gives you a menu of call progress tones as shown below. Pressing the

key corresponding to a selection will produce the sound of that signal.

Call Progress Tones

Signal                 Frequencies (Hz)              Cadence

1> Dial Tone                   350 + 440          Continuous

2> Audible Ringing Tone        440 + 480          2 seconds on 4 seconds off

3> Busy Tone                   480 + 620          .5 second on .5 second off

4> Reorder Tone(local or toll) 480 + 620          .25 second on .25 sec off

5> Reorder Tone (toll)         480 + 620          .2 second on .3 second off

6> Reorder Tone (local)        480 + 620          .3 second on .2 second off

(The reorder tone you get depends on the type of switching equipment.)

7> Special Information Tones  950,1400,1800       On in order 330 ms, off 20 ms

8> Call-Waiting Tone           440                Single 500 ms pulse

9> Low Tone                    480 + 620          Varies according to use

0> High Tone                   480                Varies according to use

A> Off Hook Warning     (1400+2060+2450+2600)     .1 sec on .1 sec off

B> Confirmation Tone   (480+620) On 200ms/off100ms/on 400ms/off 500ms/dial tone

C> Operator Emerg. Interrupt   440                2 secs on

D> Recorder Warning Tone      1400                .5 second every 15 seconds

E> TSPS Coin Control Wink     2600           33ms on/33ms off/33ms on/100ms off

F> TSPS Coin Collect           700 + 1100         Wink then on 1 second

G> TSPS Coin Return           1100 + 1700         Wink then on 1 second

H> TSPS Ringback               700 + 1700         Wink then on 2 seconds

F6 - Load dial string from disk

You can set up dial strings in advance, as simple or complex as you like,

and load them into the program with this command. A dial string can contain up

to 4000 characters.

F7 - Save dial string to disk

Prompts for a filename and saves the currently active dial string to disk.

F8 - Rotary bluebox (toggle)

Single Frequency (SF) signaling.

This is really a piece of history. Phone phreaking did not start with the

Blue Box as it has come to be known. It was started by a group of blind kids

whistling off on ancient switches that used SF (Single Frequency) signaling.

This works on a principle similar to pulse dialing with an old rotary dial

phone. The on hook breaks (1 break for 1, 2 breaks for 2, etc.) produced by the

rotary dial are replaced with short bursts of 2600 Hz. The way the kids used

this was to whistle off with 2600 Hz just as was done later with the Blue Box.

They literally did this by whistling with their mouths. When they heard the

"chirp-kerchunk" indicating seizure of a trunk, they whistled the digits of the

number they wanted to call.

Early Blue Boxes were sometimes equipped with a rotary dial and a 2600 Hz

oscillator to do this kind of phreaking.

I have always been impressed by the talent of those earliest phone

phreaks. I could never get my own "whistler" up to 2600 Hz.

To get the flavor of how it all really started, check out the Rotary Blue

Box.

F9 - Setup user key

An input screen will be displayed which lets you set up a sequence of

tones which will be attached to the "+" key. The table below shows the setup to

produce the tones for a "quarter". Frequencies are entered in Hertz. Mark is

the duration of the tones. Space is the duration of the silent period after the

tones for that line are played. Mark and Space are in milliseconds. The

sequence stops when a zero is encountered in the field for Freq1 or, if the

table is full, when all the tones have been played.

To move forward through the fields press TAB or ENTER. To backup press

SHIFT TAB. To move down one field press DOWN ARROW. To move up one field press

UP ARROW.

�������������[ Setup User Key ]�����������Ŀ

� Freq1  Freq2  Freq3  Freq4  Mark   Space �

� 2200   1700   0      0      33     33    �

�                                          �

� 2200   1700   0      0      33     33    �

�                                          �

� 2200   1700   0      0      33     33    �

�                                          �

� 2200   1700   0      0      33     33    �

�                                          �

� 2200   1700   0      0      33     33    �

�                                          �

� 0      0      0      0      0      0     �

�                                          �

� 0      0      0      0      0      0     �

�                                          �

� 0      0      0      0      0      0     �

�                                          �

� 0      0      0      0      0      0     �

�                                          �

� 0      0      0      0      0      0     �

�                                          �

� 0      0      0      0      0      0     �

��������������������������������������������

F10 - Recall last dial string

The last active dial string is recalled as if it were typed in again.

DIAL STRING FORMATS FOR P80BOX

Spaces have been injected into dial strings to aid clarity. They have no

effect on their interpretation by the program. All non-dialable characters

except delay commands are ignored. See the section titled HOW BLUE BOXING WAS

DONE for more detailed instructions on placing Blue Box calls.

BLUE BOX CALL IN THE CONTINENTAL U.S.:

FORMAT: (2600 Hz for approx. 1 sec), (Delay approx. 1 second), then send

KP + Area Code + 7 digit phone number + ST

P80BOX DIAL STRING: Assuming the number to be called is (515) 555-1234

D,K 515 555 1234 S

With everything set to program defaults, this is how the above dial string

will be interpreted by the program.

D = Send 2600 Hz for 1 second

, = Delay 1 second

K = Send KP signal for 100 milliseconds

515 555 1234 = Send MF (Multifrequency) digits 70 milliseconds on with 70

ms silent period between digits.

S = Send STart signal for 70 ms

If the number you are calling is in the local dialing area for the

switching office you are controlling you would leave out the area code.

BLUE BOX CALL TO AN OVERSEAS NUMBER:

FORMAT: (2600 Hz for approx. 1 sec), (Delay approx. 1 second)

1st stage outpulsing: KP + 011 + Paired Country Code + ST

Wait about 5 seconds for the overseas dialtone

2nd stage outpulsing: KP + Country Code + City Code + Phone Number + ST

P80BOX DIAL STRING: To call 246-8091 in London, England

D,K 011044 S,,,,,,K 044 1 2468091 S

This will be executed by the program as the following sequence:

D = Send 2600 Hz for 1 second

, = Delay 1 second

K = Send KP for 100 ms

011044 = Send 011 and paired country code as MF digits (70ms on/70ms off)

S = Send ST

,,,,,, = Delay 6 seconds allowing time for international dial tone

K = Send KP

044 1 2468091 = Send MF digits

S = Send ST

MAKING A RED BOX CALL

FORMAT: The required amount of money in nickels, dimes and quarters is

inserted into a coin slot.

P80BOX DIAL STRING: To simulate $1.15

QQQQIN

The program will generate tones for 4 quarters, 1 dime, 1 nickel.

MAKING A STANDARD DTMF CALL:

FORMAT: 1+(A/C)+7 digit number or just 7 digit number for local call.

P80BOX DIAL STRING: To call 555-1234

[5551234]

The digits enclosed in square brackets will be sent as standard DTMF

tones.

TECHNICAL SPECIFICATIONS

These are the standard DTMF (Touch Tone) frequencies in two useful

formats. The Bell Operating Company frequency tolerance is � %1.5. The Silver

Box consists of the A, B, C and D keys. The legitimate use of those keys is on

a military telephone network called Autovon, in amateur radio, and for internal

telephone company testing.

Digit   Frequencies      LOW

-----   ------------     TONE    HIGH TONE GROUP (HZ)

1       697 + 1209     GROUP

2       697 + 1336     (HZ)   1209  1336  1477  1633

3       697 + 1477

4       770 + 1209     697     1     2     3     A - Flash

5       770 + 1336     770     4     5     6     B - Flash override priority

6       770 + 1477     852     7     8     9     C - Priority communication

7       852 + 1209     941     *     0     #     D - Priority override

8       852 + 1336

9       852 + 1477

0       941 + 1336

*       941 + 1209

#       941 + 1477

A       697 + 1633

B       770 + 1633

C       852 + 1633

D       941 + 1633

Multifrequency signaling tones used for control of telephone switching

(Blue Box). Bell Operating Company frequency tolerance = � %1.5.

Digit          Frequencies (Hz)

-----          ----------------

1               700 + 900

2               700 + 1100

3               900 + 1100

4               700 + 1300

5               900 + 1300

6              1100 + 1300

7               700 + 1500

8               900 + 1500

9              1100 + 1500

0              1300 + 1500

-------------------------------

CODE 11              700 + 1700  Inward operator (European)

CODE 12              900 + 1700  Delayed-ticketing or suspended-

call operator (European)

KP (Key Pulses)     1100 + 1700  Start of digit transmission

for a national call.

KP2                 1300 + 1700  Start of digit transmission

for an international call from

an intermediate (transit) exchange.

ST (Start)          1500 + 1700  End-of-digit transmission.

Disconnect          2600 � 30 Hz

The Red Box frequencies are 2200 Hz + 1700 Hz. The Bell Operating Company

frequency tolerance for these is � %1.5. Red boxing consists of simulating the

tones produced when coins are deposited in a pay phone.

5 cents - 1 beep, 66 milliseconds duration.

10 cents - 2 beeps, each 66 milliseconds duration with 66 millisecond

pause between beeps.

25 cents - 5 beeps, each 33 milliseconds duration with a 33 millisecond

pause between beeps.

The Green Box tones are used by TSPS operators.

TSPS Coin Control Wink    2600           33ms on 33ms off 33ms on 100ms off

TSPS Coin Collect          700 + 1100    Wink then on 1 second

TSPS Coin Return          1100 + 1700    Wink then on 1 second

TSPS Ringback              700 + 1700    Wink then on 2 seconds

These are the frequencies for the European Break signal which seems to be

used by European phone phreaks to start a calling sequence the way 2600 Hz is

used in the U.S. All I know about it other than the frequencies is that it

appears in European Blue Box programs and that European phones phreaks are Blue

Boxing into underground bulletin boards in the U.S. to this day.

2390 + 2600  150 milliseconds

delay 165 ms

2410 + 2390  225 ms

delay at least 600 ms

HOW BLUE BOXING WAS DONE

"Whistling off", "beeping off", and "blowing 2600 Hz" all mean the same

thing - sending a 2600 Hz tone down a phone line for approximately 1 second. A

"blowable number" means a toll free number like an 800 number or Canadian

directory assistance which when called and fed 2600 Hz results in the seizure

of a trunk in a switching office. All Blue Box calls begin with seizure of a

trunk in this way.

BLUE BOXING WITHIN THE CONTINENTAL U.S.

Boxing a call to anywhere in the Continental United States was so simple I

don't seem to have made any personal notes on the subject. When reviewing my

notes in preparation for this doc file, all I found were a few examples I had

hard coded into a computer program. In any case, this is how it was done.

Though these instructions are given in the present tense, they are spoken from

a viewpoint in the mid 80's.

1. Go to a pay phone. Never do this from home. This is the most important

step in this procedure.

2. Call a "blowable number". It is not necessary to wait for the called

number to ring. Just listen for the click or series of clicks

indicating call completion. If it rings, fine, but Blue Boxing is a

class act and beeping off before the called number rings is classier

and more fabian than waiting until after.

3. Send 2600 Hz for approximately 1 second.

4. Listen for the "chirp-kerchunk" indicating a trunk seizure. If you have

set up an automated procedure by means of a portable computer or a

tape recording of tones produced by a computer, provide a 1 second

delay after the 2600 Hz tone.

5. Send the MF digits in this format: KP+(Area Code)+(7 digit number)+ST.

The timing of the MF digits is not super critical but if you can

control it precisely, send KP for 100 ms; delay 70 ms; send the rest of

the digits including ST at 70 ms with 70 ms interdigit silence.

BLUE BOXING TO OVERSEAS DESTINATIONS

Boxing a call to an overseas number requires more knowledge than that

required for calls within the Continental U.S. To give you this knowledge, I am

including the original notes on my research and field experiments on this

subject. I could call toll free to anywhere in the world that had a telephone.

This is the information I used to do it.

===============Start of Notes on Blue Boxing to Overseas Numbers===============

Notes on Overseas Dialing

Overseas dialing is done in two stages of outpulsing. The first stage

routes to an overseas sender and uses 011, which is the international access

code for International Direct Distance Dialing (IDDD) plus the paired country

code. If the country code is two digits, the paired country code can be

derived by adding a "0" to the left of the country code. Example: The country

code for England is 44. The paired country code would be 044. First stage

outpulsing for England would then be: KP-011044-ST. If the country code

contains three digits, the paired country code cannot be derived in this way

and must be looked up. Example: The country code for Guam is 671. The paired

country code is 067. First stage outpulsing for Guam would be KP-011067-ST.

Second example: The country code for Cyprus is 357. The paired country code is

087. It is a rule that a paired country code must never be the same as any

country code.

About five seconds after the STart pulse, an international dial tone will

be heard. This will time out to a reorder in about ten seconds.

When the dial tone is heard, the system is ready to accept the second

stage of pulsing in the format: KP-country code-city code-digits-ST. At this

stage it is the country code not the paired country code which is used.

Use the paired country codes when calling inward operators.

Some toll offices are screened against 011 coming in on a long distance

trunk. In that case precede the 011 with the area code which would apply for

that toll office. Example: for a toll office in Gainsville, FL use

KP-904+011+paired CC-ST.

Another way to reach the overseas senders is to call them directly with

KP-sender number-ST. If this doesn't work add the area code of the sender.

Example: KP-904185-ST.

This is a list of international centers with their area codes.:

A/C  Sender   Location

---  ------   --------

914   182   White Plains, NY

212   183   New York, NY

412   184   Pittsburgh, PA

904   185   Jacksonville, FL

415   186   Sacramento, CA

303   187   Denver, CO

212   188   New York (same sender as 183)

The routing for a particular country can be found by dialing normally

(pulse or touch-tone) 011+CC+000+enough digits to add up to a total of seven

including the country code. Example: 011+44+00011. You will get a recording. At

the end of the recording, the area code of the international center will be

given. The sender used to call a particular country can vary depending on the

area of the country from which the call is originated. An international call

can sometimes be completed through the wrong sender, but this causes a print

out that will later be investigated to find out which CO it came from. To find

the correct routing when pulsing through any particular toll office use

KP+paired CC+000+ST. For example, KP-011044000-ST would give the same result as

dialing normally 011-44-00011 if you were dialing it in the area where the toll

office is located.

The first digit of a country code is the world region in which that

country is located. The world regions are: 1--North America, 2--Africa, 3 and

4--Europe, 5--South and Central America, 6--South Pacific, 7--Union of Soviet

Socialist Republics (U.S.S.R.), 8--Far East, 9--Middle East and South-East

Asia.

Note 1. KP2 is not used in first or second stage outpulsing when calling any

country in the IDDD network.

Note 2. Public telephones are interfaced to TSPS (Traffic Service Position

System). If you call an 800 number and whistle off using 2600 Hz, the distant

toll office sends a wink back signal (a short on-hook) indicating it is ready

to receive pulsing. TSPS responds to this wink back by printing out the

original number called, the number called from, and the number MFed after the

wink back. This print out goes to the billing and security departments.

General Notes on Overseas Senders and Routing Codes

(June 1984)

From Canadian Directory Assistance (514) call Montreal sender with KP 188

ST. Using this sender these countries can be called:

England (Satellite)

Australia (Satellite)

Egypt (Cable only)

Philippines (Satellite)

Could not reach Thailand or Japan from this sender. Canadians cannot

direct dial these countries. They have to go through an overseas operator in

Vancouver.

Guam can be reached by satellite by beeping off 514 directory assistance

and calling the sender in Jacksonville with KP 904185 ST.

The following senders could be reached through a PBX extender in Orlando,

Florida. After getting an outside line through the extender any number can be

called. After a series of clicks, blow 2600 Hz. This will give control of a

tandem in the Orlando area.

A/C  Sender   Location

---  ------   --------

914   182   White Plains

212   183   New York

412   184   Pittsburgh

904   185   Jacksonville

303   187   Denver

These senders could not be reached by this method:

415   186   Oakland

514   188   Montreal

European countries can be reached by beeping off an 800 number and keying

KP 011 Derived CC ST. This will automatically connect you to the right sender

for that country code and will give you an overseas dial-tone for ten seconds.

For example to call the weather recording in England, the routing would be KP

011044 ST then KP 044 1 246 8091 ST.

PAIRED COUNTRY CODES

This is a list of paired country codes for use in first stage outpulsing

on overseas calls. For two digit country codes simply add a zero. Example:

The country code for England is 44. The paired country code is 044. Paired

country codes that cannot be derived by this simple method are listed

below.

Country                Country Code  Paired Code

-------                ------------  -----------

Algeria                           213           013

American Samoa                    684           284

Bahrain                           973           073

Belize                            501           111

Bolivia                           591           991

Brune                             773           180

Cameroon                          237           077

Costa Rica                        506           806

Cyprus                            357           087

Ecuador                           593           293

El Salvador                       503           003

Ethiopia                          251           059

Fiji                              679           879

Finland                           358           088

French Antilles                   596           896

French Polynesia (Tahiti)         689           289

Gabon                             241           025

Gibraltar                         350           050

Guam                              671           067

Guatemala                         502           022

Guyana                            592           892

Haiti                             509           887

Honduras                          504           884

Hong Kong                         852           692

Iceland                           354           854

Iraq                              964           294

Ireland                           353           083

Israel                            972           072

Ivory Coast                       225           285

Jordan                            962           282

Kenya                             254           074

Kuwait                            965           015

Lesotho                           266           186

Liberia                           231           851

Libya                             218           018

Luxembourg                        352           292

Malawi                            265           096

Marisat Atlantic                  871           101

Marisat Pacific                   872           102

Marisat Indian Ocean              873           103

Morocco                           212           012

Namibia                           264           194

Netherlands Antilles              599           099

New Caledonia                     687           287

Nicaragua                         505           975

Nigeria                           234           014

Oman                              968           068

Panama                            507           247

Papua New Quinea                  675           875

Paraguay                          595           295

Portugal                          351           281

Qatar                             974           174

Saipan                            670           071

Saudi Arabia                      966           990

Senegal                           221           021

St. Pierre/Miguelon               508           104

Suriname                          597           097

Swaziland                         268           168

Taiwan                            886           006

Tanzania                          255           075

Tunisia                           216           016

Uganda                            256           876

United Arab Emirates              971           291

Uruguay                           598           288

USSR                                7           007

Yemen Arab Republic               967           297

Zambia                            260           008

Zimbabwe (Rhodesia)               263           283

Notes: The Marisat codes are used when calling ships directly. Single stage

outpulsing is used for calls to Mexico in this format: KP-180-City

Code-digits-ST. To call an inward operator in Mexico use KP-190-City

Code-09-ST. For directory assistance use KP-190-City Code-01-ST.

================End of Notes on Blue Boxing to Overseas Numbers================

SILVER BOXING (a.k.a. WHITE BOXING)

I didn't experiment much with this and took no notes on it. I checked it

out once or twice with a young phreak who lived in the midwest. The way it

worked was two phone phreaks in different parts of the country would call

directory assistance in an area that was still on the old 4A switch.

Immediately after dialing the last digit you would hold down the 'D' key on the

silver box. Instead of getting a directory assistance operator you would hear a

pulsing dial tone. Then one caller would send a Touch Tone 6 and the other

would send a Touch Tone 7. The two would then be connected to each other

through a test loop where they could talk indefinitely toll free.

RED BOXING

Red Boxing is a method of making free calls by simulating the tones

produced by dropping coins into a payphone. The specs for these tones are given

above under TECHNICAL SPECIFICATIONS. I experimented intensively with this for

about three weeks. My main method for producing the tones was to make a

cassette recording of an endless string of quarter tones and then play them

into a payphone receiver as needed.

I have the uncertain honor of having invented Red Boxing with whistles.

The whistles were made from aluminum tubing available in hobby shops. They were

1/4 inch in diameter by 4 inches long and were tuned by means of a wooden dowel

rod which fit snugly inside. I tuned one to 2200 Hz and another to 1700 Hz by

adjusting the dowel rod in and out while listening for a zero beat against a

known signal source (a computer). I taped the two whistles together snugly with

masking tape. After very little practice, I found that I could whistle nickels,

dimes and quarters that were accepted by a payphone.

I made a set of these whistles for my Silver Boxing friend, mentioned

above, and mailed them to him. He used them every day for a week to call me up

and demonstrate this new technique for "blowing" money.